ENES-222 · Signals & Systems

Lecture 22 Notes

Periodic Inputs & Cascaded Filters

Thesis of the lecture: Lecture 21 established H(e^{jΩ}) as a filter's fingerprint at a single frequency; this lecture spends that fingerprint four ways — on a real sinusoid with phase, on a damped/growing exponential (by moving off the unit circle to z=re^{jΩ₀}), on a whole periodic signal at once (by sampling H at the DFT frequencies and multiplying), and on a cascade of two filters (by multiplying system functions). All four are the same eigen-property from Lecture 21, applied to richer and richer classes of input — that throughline is worth stating explicitly, since on the page these can look like four unrelated tricks. Time plan: about 58 minutes of core slides; Solved Examples A and B (slides 24–25) are optional reading if the class runs long.

## Slide 1  —  Title — Periodic Inputs & Cascaded Filters

- Subtitle: periodic in, periodic out; the DFT of the output is the DFT of the input times samples of the frequency response; cascade ⇒ multiply system functions.

- Ω is discrete-time frequency (rad/sample) throughout; ω is reserved for continuous time. The notebook’s code names the variable omega0 for the DT frequency, and its cascade figure labels the axis omega/(2pi): read both as Ω.

Timing: ≈ 0 min (part of the 5-min intro)

## Slide 2  —  Objectives

- Four skills: real sinusoid in → scaled and phase-shifted out (amplitude A, frequency Ω₀, phase φ); damped exponentials via z = re^{jΩ₀}; periodic inputs via Y[k] = H(e^{j2πk/L})X[k]; cascades H = H₁H₂.

- Roadmap: 1 ↔ slides 3, 9–10; 2 ↔ slides 4–8; 3 ↔ slides 11–20; 4 ↔ slides 21–25. Slide 26 collects the common mistakes.

Timing: ≈ 1 min

## Slide 3  —  Concept — Real Sinusoidal Inputs

- Step 1: cosθ = ½(e^{jθ} + e^{−jθ}) with θ = Ω₀n + φ, so x[n] = ½e^{jφ}e^{jΩ₀n} + ½e^{−jφ}e^{−jΩ₀n}.

- Step 2: apply the L21 eigen-property to each term. With real b, H(e^{−jΩ₀}) = H*(e^{jΩ₀}).

- Step 3: with w = H(e^{jΩ₀})e^{j(Ω₀n+φ)} = |H|e^{j(Ω₀n+φ+∠H)}, y[n] = ½(w + w*) = Re{w} = |H|cos(Ω₀n + φ + ∠H).

- Frequency is preserved, which is why FIR filters are frequency-selective.

Timing: ≈ 3 min

## Slide 4  —  Concept — Oscillating Exponentials

- x[n] = rⁿcos(Ω₀n + φ). Since zⁿ = rⁿe^{jΩ₀n} for z = re^{jΩ₀}, x[n] = Re{e^{jφ}zⁿ}. Real b lets Re pass through the filter: y[n] = Re{e^{jφ}H(z)zⁿ} = |H(z)| rⁿ cos(Ω₀n + φ + ∠H(z)).

- Reduces to slide 3 when r = 1; r < 1 decays, r > 1 grows.

Common error: Evaluating H at e^{jΩ₀} instead of at z = re^{jΩ₀}.

Timing: ≈ 2 min

## Slide 5  —  Worked Example — Damped Cosine, Setup

- Input x[n] = 2^{−n}cos(πn/3 + π/4); filter H(z) = 1 + 2z⁻¹ + 2z⁻² + z⁻³ (b = (1,2,2,1), same as the notebook).

- Step 1: 2^{−n} = (1/2)ⁿ so r = 1/2, Ω₀ = π/3, φ = π/4, and z = ½e^{jπ/3}.

- Step 2: z⁻¹ = 2e^{−jπ/3}, z⁻² = 4e^{−j2π/3}, z⁻³ = 8e^{−jπ}.

- Step 3: H(z) = 1 + 2·2e^{−jπ/3} + 2·4e^{−j2π/3} + 8e^{−jπ} = 1 + 4e^{−jπ/3} + 8e^{−j2π/3} + 8e^{−jπ}.

Timing: ≈ 3 min

## Slide 6  —  Worked Example — Damped Cosine, Result

- Step 4: 4e^{−jπ/3} = 2 − j2√3; 8e^{−j2π/3} = −4 − j4√3; 8e^{−jπ} = −8.

- Step 5: real part 1 + 2 − 4 − 8 = −9; imaginary part −2√3 − 4√3 = −6√3 ≈ −10.392, so H(z) = −9 − j6√3.

- Step 6: |H| = √(81 + 108) = √189 ≈ 13.748. Third quadrant: ∠H = −π + arctan(6√3/9) = −3.1416 + 0.8571 ≈ −2.285.

- Step 7: y[n] = 13.748·2^{−n}cos(πn/3 + π/4 − 2.285) = 13.748·2^{−n}cos(πn/3 − 1.499) (π/4 = 0.785).

Common error: Reading the angle as +0.857 (the bare arctangent) instead of −π + 0.857: the point −9 − j10.39 is in the third quadrant.

Timing: ≈ 3 min

## Slide 7  —  Visual — Formula vs. Filter (notebook figure)

- Figure is notebook cell 2 of the Lecture 22 notebook: gray = input, blue = direct filter output, red dashed = formula.

- Point at n ≥ 3 where blue and red coincide and decay together; at n = 0..2 they differ (formula ≈ 6.2 at n = 1, filter ≈ 1.3). The next slide explains why.

Timing: ≈ 2 min

## Slide 8  —  Concept — Transient Versus Steady State

- The filter has order M = 3 (four taps), so y[n] needs x[n], x[n−1], x[n−2], x[n−3]. The notebook starts the input at n = 0, so for n < 3 some of those samples are zero; the formula assumes x exists for all n.

- Table on the slide (filter / formula): n=0: 0.7071 / 0.9845; n=1: 1.2848 / 6.1838; n=2: 0.9139 / 2.8458; n=3: −0.1231 / −0.1231; n=4: −0.7730 / −0.7730. The notebook prints max |difference| = 4.90 over the early samples (at n = 1).

- Hand check at n = 3: x[0..3] = 0.7071, −0.1294, −0.2415, −0.0884; y[3] = −0.0884 + 2(−0.2415) + 2(−0.1294) + 0.7071 = −0.0884 − 0.4830 − 0.2588 + 0.7071 = −0.1231.

Rule: With M+1 taps the first M outputs are the start-up transient; from n = M the prediction is exact.

Timing: ≈ 2 min

## Slide 9  —  Practice — Undamped Cosine Through the Filter

- b = (1,2,2,1), x[n] = cos(πn/3 + π/4) (the notebook’s “Your task”, r = 1). Students write H(e^{jπ/3}) with all four terms and use the exact values of e^{−jπ/3}, e^{−j2π/3}, e^{−jπ}.

Hint to give: Ask which axis H lies on before anyone reaches for an arctangent.

Timing: ≈ 4 min (students work; show slide 10 after ~3 min)

## Slide 10  —  Solution — Undamped Cosine

- H(e^{jπ/3}) = 1 + 2e^{−jπ/3} + 2e^{−j2π/3} + e^{−jπ} = 1 + (1 − j√3) + (−1 − j√3) + (−1). Real: 1 + 1 − 1 − 1 = 0. Imaginary: −√3 − √3 = −2√3.

- So H = −j2√3 ≈ −j3.464: |H| = 2√3 = 3.464 and ∠H = −π/2 (negative imaginary axis).

- y[n] = 3.464cos(πn/3 + π/4 − π/2) = 3.464cos(πn/3 − π/4).

Check: y[0] = x[0] + 2x[−1] + 2x[−2] + x[−3] = 0.7071 + 2(0.9659) + 2(0.2588) − 0.7071 = 2.4495, and 3.464cos(−π/4) = 2.4495. Also H = e^{−j3Ω/2}(2cos(3Ω/2) + 4cos(Ω/2)): at Ω = π/3 the first cosine is 0 and 4cos(π/6) = 3.464, phase −π/2.

Timing: ≈ 3 min

## Slide 11  —  Concept — Periodic Inputs via DFT

- X[k] = Σ_{n=0}^{L−1} x[n]e^{−j2πkn/L} is the DFT of one period; inverse DFT x[n] = (1/L)Σ X[k]e^{jΩ_k n}, Ω_k = 2πk/L.

- Each term is a complex exponential, so the filter multiplies it by H(e^{jΩ_k}); hence Y[k] = H(e^{j2πk/L})X[k]. Period L in ⇒ period L out (transient ignored).

- Code: X = fft(x); H = fft(b,L); Y = H .* X; y = ifft(Y).

Timing: ≈ 3 min

## Slide 12  —  Derivation — Why Multiplying DFTs Works

- Three lines: (1) inverse DFT writes x as a sum of L exponentials; (2) each exponential z^n with z = e^{jΩ_k} comes out multiplied by H(e^{jΩ_k}), linearity handles the sum; (3) the result has the inverse-DFT form with coefficients Y[k] = H(e^{jΩ_k})X[k].

- H(e^{jΩ_k}) = Σ_{m=0}^{M} b_m e^{−j2πkm/L}: this is the DFT of b zero-padded to length L, hence fft(b, L).

Common error: Needs L ≥ M+1. If the filter is longer than the period, b_m must be folded into index m mod L (not truncated), because e^{−j2πkm/L} repeats every L.

Timing: ≈ 3 min

## Slide 13  —  Practice — Period of a Three-Cosine Sum

- f₁ = 1/8, f₂ = 3/20, f₃ = 5/12 (cycles/sample, Ω = 2πf). Instruction on the slide: write f = p/q in lowest terms; period is q.

Common error: Using L = 1/f. That works only when p = 1; for 3/20, 1/f = 6.67 is not an integer.

Timing: ≈ 3 min

## Slide 14  —  Solution — L = 120

- cos(2π(p/q)n) repeats when (p/q)L is an integer; with gcd(p,q) = 1 the smallest such L is q. So L₁ = 8, L₂ = 20 (gcd(3,20) = 1), L₃ = 12 (gcd(5,12) = 1).

- 8 = 2³, 20 = 2²·5, 12 = 2²·3; L = lcm = 2³·3·5 = 120. Check 120/8 = 15, 120/20 = 6, 120/12 = 10, all integers.

- Bins k = L·f: 120/8 = 15, 120·3/20 = 18, 120·5/12 = 50; with mirrors L − k: {15,105}, {18,102}, {50,70}. Six nonzero bins out of 120.

- DT factoid: a finite sum of periodic DT signals is always periodic (not true in CT).

Timing: ≈ 3 min

## Slide 15  —  Worked Example — Length-4 Input, DFT of x

- Filter y[n] = x[n] − 4x[n−1] + x[n−2], b = (1,−4,1)ᵀ; one period x = (2,1,−3,5)ᵀ, L = 4 (same as the notebook). With L = 4, e^{−j2πkn/4} = (−j)^{kn}.

- X[0] = 2+1−3+5 = 5. X[1] = 2 + 1(−j) + (−3)(−1) + 5(j) = 5 + 4j. X[2] = 2 − 1 − 3 − 5 = −7. X[3] = 2 + j + 3 − 5j = 5 − 4j = X*[1] (x real).

- Powers used: (−j)² = −1, (−j)³ = j.

Timing: ≈ 3 min

## Slide 16  —  Worked Example — Sample H, Multiply

- H[k] = 1 − 4(−j)^k + (−j)^{2k}: H[0] = 1−4+1 = −2; H[1] = 1 − 4(−j) − 1 = 4j; H[2] = 1 + 4 + 1 = 6; H[3] = 1 − 4j − 1 = −4j.

- Y[k] = H[k]X[k]: Y[0] = (−2)(5) = −10; Y[1] = (4j)(5+4j) = 20j − 16; Y[2] = 6(−7) = −42; Y[3] = (−4j)(5−4j) = −20j − 16. Used j·j = −1.

Timing: ≈ 2 min

## Slide 17  —  Worked Example — Inverse DFT and Check

- y[n] = ¼ΣY[k]jᵏⁿ. y[0] = ¼(−10 − 16 + 20j − 42 − 16 − 20j) = −21; y[1] = ¼(−10 + (−16j − 20) + 42 + (16j − 20)) = −2; y[2] = ¼(−10 + (16 − 20j) − 42 + (16 + 20j)) = −5; y[3] = ¼(−10 + (16j + 20) + 42 + (−16j + 20)) = 18.

- Independent check by circular convolution y[n] = b₀x[n] + b₁x[(n−1) mod 4] + b₂x[(n−2) mod 4]: y[0] = 2 − 20 − 3 = −21; y[1] = 1 − 8 + 5 = −2; y[2] = −3 − 4 + 2 = −5; y[3] = 5 + 12 + 1 = 18. Both give (−21, −2, −5, 18)ᵀ, matching the notebook.

- Pointwise multiplication of DFTs ⇔ circular convolution.

Timing: ≈ 3 min

## Slide 18  —  Visual — Three Periods (notebook figure)

- Figure is notebook cell 4 of the Lecture 22 notebook. Blue circles: input 2, 1, −3, 5 repeating. Red squares: output −21, −2, −5, 18, drawn 0.2 to the right so stems do not overlap.

- Point out the same period 4 and how large the output is compared with the input (b₁ = −4 amplifies).

Timing: ≈ 1 min

## Slide 19  —  Worked Example — DC Offset, DFT and H

- Filter y[n] = x[n] − x[n−2], b = (1,0,−1)ᵀ; one period x = (4,3,2,3)ᵀ = 3 + cos(πn/2), L = 4.

- X[0] = 12; X[1] = 4 + 3(−j) + 2(−1) + 3(j) = 2; X[2] = 4 − 3 + 2 − 3 = 0; X[3] = 4 + 3j − 2 − 3j = 2.

- H[k] = 1 − (−j)^{2k} = 1 − (−1)^k, so H = (0, 2, 0, 2); then Y = (0·12, 2·2, 0·0, 2·2) = (0, 4, 0, 4). The DC bin is killed because H[0] = 0.

Timing: ≈ 3 min

## Slide 20  —  Worked Example — DC Offset, Inverse and Figure

- y[n] = ¼(4jⁿ + 4(−j)ⁿ) = 2cos(πn/2), so y = (2, 0, −2, 0)ᵀ. Circular check: y[0] = x[0] − x[2] = 2; y[1] = x[1] − x[3] = 0; y[2] = x[2] − x[0] = −2; y[3] = x[3] − x[1] = 0.

- Figure (generated for this deck): top, three periods of x (blue) and y (red, shifted 0.2); bottom, bars |X|, |H|, |Y| at bins 0..3. Bin 0 (height 12) disappears, bins 1 and 3 (Ω = ±π/2) double (2 → 4).

Takeaway: Filtering a periodic signal = a gain per DFT bin; here a gain of 0 at DC and 2 at Ω = ±π/2.

Timing: ≈ 2 min

## Slide 21  —  Concept — Cascade of Filters

- Proof: x[n] = zⁿ → filter 1 gives H₁(z)zⁿ → filter 2 scales by H₂(z), giving H₂(z)H₁(z)zⁿ. So H = H₁H₂ = H₂H₁.

- Order is irrelevant because H₁(z) and H₂(z) are ordinary numbers for each z (unlike matrices). Coefficients of the cascade are the linear convolution of b⁽¹⁾ and b⁽²⁾.

Timing: ≈ 2 min

## Slide 22  —  Worked Example — Cascade of Two FIRs

- b⁽¹⁾ = (1,2,2,1)ᵀ, b⁽²⁾ = (1,−4,1)ᵀ; let u = z⁻¹: H₁ = 1 + 2u + 2u² + u³, H₂ = 1 − 4u + u².

- Partial products: 1·H₂ = 1 − 4u + u²; 2u·H₂ = 2u − 8u² + 2u³; 2u²·H₂ = 2u² − 8u³ + 2u⁴; u³·H₂ = u³ − 4u⁴ + u⁵.

- Collect: u⁰: 1; u¹: −4+2 = −2; u²: 1−8+2 = −5; u³: 2−8+1 = −5; u⁴: 2−4 = −2; u⁵: 1. H(z) = 1 − 2z⁻¹ − 5z⁻² − 5z⁻³ − 2z⁻⁴ + z⁻⁵, so y[n] = x[n] − 2x[n−1] − 5x[n−2] − 5x[n−3] − 2x[n−4] + x[n−5].

Consistency check: At z = 1: H₁(1)H₂(1) = 6·(−2) = −12 and 1−2−5−5−2+1 = −12.

Timing: ≈ 3 min

## Slide 23  —  Visual — Cascade in Frequency (notebook figure)

- Figure is notebook cell 6 of the Lecture 22 notebook; axis is Ω/2π (the notebook labels it omega/(2pi), meaning Ω/2π).

- Left: |H₁| (blue, 0–6), |H₂| (red, 2–6, since H₂ = e^{−jΩ}(2cosΩ − 4)) and their product (green dashed): peak 6·2 = 12 at Ω = 0, and H₁’s three nulls survive.

- Right: the six-tap filter (1,−2,−5,−5,−2,1) computed directly lies on the product curve.

Timing: ≈ 1 min

## Slide 24  —  Solved Example A — b = (1,2,1), x[n] = cos(πn/2)

- Ω₀ = π/2 rad/sample, φ = 0. Step 1: H(e^{jΩ}) = 1 + 2e^{−jΩ} + e^{−j2Ω} = e^{−jΩ}(e^{jΩ} + 2 + e^{−jΩ}) = e^{−jΩ}(2 + 2cosΩ).

- Step 2: H(e^{jπ/2}) = e^{−jπ/2}(2 + 2cos(π/2)) = (−j)(2) = −2j, so |H| = 2, ∠H = −π/2.

- Step 3: y[n] = 2cos(πn/2 − π/2) = 2sin(πn/2), using cos(θ − π/2) = sinθ.

- Check with x = …, 0, 1, 0, −1, 0, … at n = −1..3: y[1] = x[1] + 2x[0] + x[−1] = 2 = 2sin(π/2); y[3] = x[3] + 2x[2] + x[1] = −2 = 2sin(3π/2).

Timing: ≈ 3 min if time permits, otherwise assign as reading

## Slide 25  —  Solved Example B — Cascade H₁ = 1 − z⁻¹, H₂ = 1 + z⁻¹

- Step 1: H(z) = (1 − z⁻¹)(1 + z⁻¹) = 1 − z⁻² (difference of squares with u = z⁻¹).

- Step 2: with z = e^{jΩ}, H = 1 − e^{−j2Ω} = e^{−jΩ}(e^{jΩ} − e^{−jΩ}) = 2j e^{−jΩ} sinΩ (Euler), so |H| = 2|sinΩ|.

- Step 3: coefficients of z^{−k}: b₀ = 1, b₁ = 0, b₂ = −1, so y[n] = x[n] − x[n−2].

- Nulls: sinΩ = 0 at Ω = 0, π: H(e^{j0}) = 1 − 1 = 0 and H(e^{jπ}) = 1 − e^{−j2π} = 0 — rejects DC and π. (Compare slide 19: the same filter removed the DC offset.)

Timing: ≈ 3 min if time permits, otherwise assign as reading

## Slide 26  —  Common Mistakes

- 1 Damped input, undamped z: for the same filter and Ω₀ = π/3, |H(e^{jΩ₀})| = 3.46 but |H(½e^{jΩ₀})| = 13.75.

- 2 Quadrant of the angle: H = −9 − j10.39 gives ∠H = arctan(10.39/9) − π = −2.285, not +0.857.

- 3 Amplitudes multiply, phases add, frequency never changes.

- 4 Period L = lcm of the denominators of f = p/q in lowest terms, not the product and not 1/f unless p = 1.

- 5 Use fft(b, L) with L ≥ M+1 and combine element-wise, never a matrix or dot product.

- 6 Cascade: the coefficient vectors convolve; (1,2,2,1) + (1,−4,1) is wrong, the answer has 4+3−1 = 6 taps.

Timing: ≈ 2 min

## Slide 27  —  Summary — Four Facts

- 01 Sinusoidal: y = |H|A cos(Ωn + φ + ∠H). 02 Damped: z = re^{jΩ₀}, H(z) replaces H(e^{jΩ}). 03 Periodic: Y[k] = H(e^{j2πk/L})X[k]. 04 Cascade: H = H₁H₂, order-independent.

- Bridge: the cascade rule says the coefficients of the combined filter are the convolution of the two coefficient vectors. Next lecture, L23, makes that convolution precise for any input, not only for sinusoids and periodic signals.

Timing: ≈ 1 min