ENES-222 · Signals & Systems

Lecture 23 Notes

Linear Convolution & Impulse Response

Thesis of the lecture: The FIR coefficient vector b is a special case of the impulse response h[n], which by linearity and time invariance determines the response to every input via y = h*x. The lecture treats convolution as a flip-shift-dot recipe (slides 3–13), as a fast DFT computation once zero-padding prevents wrap-around (14–16), and as block/overlap-add processing for long signals (17–22). Time plan: about 55 minutes of core slides; Solved Examples A and B are optional reading.

## Slide 1  —  Title — Linear Convolution & Impulse Response

- The impulse response h[n] characterizes an LTI system completely; the output is y = h * x, a true (non-circular) convolution sum.

- Roadmap: impulse response and derivation (3–4), recipe (5–10), rules and practice (11–13), DFT implementation (14–16), block processing (17–22), extra solved examples (23–24), mistakes (25).

Timing: ≈ 0 min (intro)

## Slide 2  —  Objectives

- Recognize h[n] as the response to δ[n]; compute the linear convolution of vectors of lengths K and L; implement it as zero-padded circular convolution; block-process a long input by overlap-add.

- Map: 1 ↔ slides 3–4, 2 ↔ 5–13, 3 ↔ 14–16, 4 ↔ 17–22.

Timing: ≈ 1 min

## Slide 3  —  Impulse Response

- x[n] = δ[n] ⇒ y[n] = h[n]. For any input, y[n] = Σ_k h[k] x[n−k] = (h*x)[n]. δ[n] = 1 at n = 0, 0 elsewhere.

- For an FIR filter with coefficients b_0..b_M, h[n] = b_n for n = 0..M, zero otherwise: the b of L21–22 is h.

- Convolution is commutative, h*x = x*h.

Why it matters: One signal, h, determines the response to every input. Say this before any arithmetic.

Timing: ≈ 2 min

## Slide 4  —  Derivation — Why y = h * x

- Step 1: x[n] = Σ_k x[k]δ[n−k]; example x = (1,2,3): 1δ[n] + 2δ[n−1] + 3δ[n−2].

- Step 2: time invariance δ[n−k] → h[n−k]; linearity x[k]δ[n−k] → x[k]h[n−k].

- Step 3: add: y[n] = Σ_k x[k]h[n−k] = Σ_m h[m]x[n−m] with m = n−k.

- Numbers, h = (1,1): 1·(1,1,0,0) + 2·(0,1,1,0) + 3·(0,0,1,1) = (1,3,5,3).

Timing: ≈ 3 min

## Slide 5  —  Mechanics — Flip, Shift, Dot

- One output sample y[n]: (1) reverse x, x̃[k] = x[−k]; (2) delay by n, giving x[n−k]; (3) dot product with h. Repeat for every n.

- Entries outside the stored range are zero, so only terms with 0 ≤ n−k ≤ L−1 survive.

Timing: ≈ 2 min

## Slide 6  —  Worked Example — One Output Sample (figure generated for this deck)

- h = b = (2,−1,1,−2), x = (1,2,3,−1), n = 3. x[3−k] for k = 0..3 is x backwards: (−1, 3, 2, 1).

- Products: 2(−1) = −2; (−1)(3) = −3; 1(2) = 2; (−2)(1) = −2. Sum −2 − 3 + 2 − 2 = −5 = y[3].

- Figure: top two panels h[k], x[k]; third x[3−k]; bottom the four products (−2, −3, 2, −2).

Timing: ≈ 3 min

## Slide 7  —  Worked Example — b * x

- b = (2,−1,1,−2)ᵀ (K = 4), x = (1,2,3,−1)ᵀ (L = 4); output length 7. Same vectors as the notebook.

- y[0] = 2.  y[1] = 4 − 1 = 3.  y[2] = 6 − 2 + 1 = 5.  y[3] = −2 − 3 + 2 − 2 = −5.  y[4] = 1 + 3 − 4 = 0.  y[5] = −1 − 6 = −7.  y[6] = 2.

Consistency check: Entries sum to (Σb)(Σx) = 0·5 = 0, and 2+3+5−5+0−7+2 = 0.

Timing: ≈ 4 min

## Slide 8  —  Worked Example — Product Grid

- Grid of all b_k x_m; the product with k+m = n goes into y[n]: anti-diagonal sums give y = (2, 3, 5, −5, 0, −7, 2): y[2] = 6−2+1, y[3] = −2−3+2−2, y[4] = 1+3−4, y[5] = −1−6.

- Remark: this is polynomial multiplication, the opening topic of L24.

Timing: ≈ 3 min

## Slide 9  —  Figure — Convolution Plot (notebook cell 3)

- Three stems: h = b, x, y. Output 7 samples (indices 0–6) from 4 + 4. The zero at n = 4 is a genuine zero.

Timing: ≈ 1 min

## Slide 10  —  Output Length Rule

- Length K+L−1, activity interval [0, K+L−2]; check 4+4−1 = 7. This tells you how much to pad before circular convolution.

Timing: ≈ 1 min

## Slide 11  —  Concept — Convolution Rules

- Commutative; associative ((h₁*h₂)*x = h₁*(h₂*x), the cascade rule of L22); distributive (parallel filters, block processing); identity x*δ = x and delay x*δ[n−d] = x[n−d] (example (1,2,3)*δ[n−2] = (0,0,1,2,3)); sum rule Σy = (Σh)(Σx).

Timing: ≈ 2 min

## Slide 12  —  Practice — Convolve Two Short Vectors

- b = (1,1,1)ᵀ, x = (2,0,−1,3)ᵀ. Students predict the length (6), compute every sample, check with the sum rule.

Timing: ≈ 4 min

## Slide 13  —  Solution — b * x = (2,2,1,2,2,3)

- y[0] = 2; y[1] = 0 + 2 = 2; y[2] = −1 + 0 + 2 = 1; y[3] = 3 − 1 + 0 = 2; y[4] = 3 − 1 = 2; y[5] = 3.

- Sum rule: (Σb)(Σx) = 3·4 = 12 and 2+2+1+2+2+3 = 12. Interior samples are running sums of three inputs.

Timing: ≈ 2 min

## Slide 14  —  Convolve via Padded DFT

- b * s = [b; 0_{L−1}] ⊛ [s; 0_{K−1}]: pad both to length K+L−1 or more. Code: B = fft(b, K+L−1); S = fft(s, K+L−1); y = ifft(B .* S).

Common error: Padding only to max(K,L): the tail wraps (next slide).

Timing: ≈ 2 min

## Slide 15  —  Concept — Why Padding Is Needed (figure generated for this deck)

- Linear result c = (2,3,5,−5,0,−7,2), DFT length 4: c[n] + c[n+4]: n=0: 2+0 = 2; n=1: 3−7 = −4; n=2: 5+2 = 7; n=3: −5. Circular result (2,−4,7,−5) — verified with ifft of the length-4 DFTs.

- Figure: top linear (tail shaded), bottom circular. With N ≥ 7 nothing wraps.

Timing: ≈ 2 min

## Slide 16  —  Worked Example — Linear Convolution via the DFT

- b = (1,2), x = (3,−1,2); N = K+L−1 = 4, e^{−j2πkn/4} = (−j)^{kn}. B = (3, 1−2j, −1, 1+2j); X = (4, 1+j, 6, 1−j).

- Y = (12, (1−2j)(1+j) = 3−j, −6, (1+2j)(1−j) = 3+j).

- y[n] = ¼ΣY[k]jᵏⁿ: y[0] = 12/4 = 3; y[1] = ¼(12 + (3j+1) + 6 + (−3j+1)) = 5; y[2] = ¼(12 − 3 + j − 6 − 3 − j) = 0; y[3] = ¼(12 + (−3j−1) + 6 + (3j−1)) = 4.

- Direct check: (3, −1+6, 2−2, 4) = (3,5,0,4) ✓.

Timing: ≈ 4 min

## Slide 17  —  Block Convolution

- b * [s⁽¹⁾; s⁽²⁾] = [b*s⁽¹⁾; 0_{L₂}] + [0_{L₁}; b*s⁽²⁾]. Real-time filters use fixed-length FFTs; overlapping tails must be added.

Common error: Discarding the overlap instead of adding it.

Timing: ≈ 2 min

## Slide 18  —  Practice — Block Convolve a Length-12 Input

- b = (1,−3,−3,1)ᵀ, s = (1,−2,3,4,4,−2,7,5,1,3,−1,−4)ᵀ (notebook example). Split into two length-6 blocks, convolve each, overlap-add. Give two minutes before showing slide 19.

Timing: ≈ 4 min (set-up), solution slides follow

## Slide 19  —  Solution Step 1 — b * s⁽¹⁾

- s⁽¹⁾ = (1,−2,3,4,4,−2): 9 samples. n=0: 1. n=1: −2−3 = −5. n=2: 3+6−3 = 6. n=3: 4−9+6+1 = 2. n=4: 4−12−9−2 = −19. n=5: −2−12−12+3 = −23. n=6: 6−12+4 = −2. n=7: 6+4 = 10. n=8: −2.

- y⁽¹⁾ = (1,−5,6,2,−19,−23,−2,10,−2).

Timing: ≈ 3 min

## Slide 20  —  Solution Step 2 — b * s⁽²⁾

- s⁽²⁾ = (7,5,1,3,−1,−4): n=0: 7. n=1: 5−21 = −16. n=2: 1−15−21 = −35. n=3: 3−3−15+7 = −8. n=4: −1−9−3+5 = −8. n=5: −4+3−9+1 = −9. n=6: 12+3+3 = 18. n=7: 12−1 = 11. n=8: −4.

- y⁽²⁾ = (7,−16,−35,−8,−8,−9,18,11,−4).

Timing: ≈ 2 min

## Slide 21  —  Solution Step 3 — Overlap-Add

- y⁽²⁾ delayed by 6; total length 15; indices 6–8 overlap (K−1 = 3). Sums: n=6: −2+7 = 5; n=7: 10−16 = −6; n=8: −2−35 = −37.

- Result (1,−5,6,2,−19,−23,5,−6,−37,−8,−8,−9,18,11,−4), equal to np.convolve(b,s). Sum check (−4)(19) = −76.

Timing: ≈ 3 min

## Slide 22  —  Figure — Overlap-Add Plot (notebook cell 5)

- Blue b*s1, red b*s2 from index 6, dashed green direct result, yellow band = overlap. Inside the band the dashed value is the sum (5 at n = 6).

Timing: ≈ 1 min

## Slide 23  —  Solved Example A — Length-3 Filter, Length-5 Input

- b = (3,−2,1), x = (1,0,2,−1,4); length 7. c[0] = 3; c[1] = −2; c[2] = 6+0+1 = 7; c[3] = −3−4+0 = −7; c[4] = 12+2+2 = 16; c[5] = −8−1 = −9; c[6] = 4.

- Check: Σc = 12 = (3−2+1)(1+0+2−1+4) = 2·6. (This replaces an earlier version that repeated the slide-7 numbers.)

Timing: ≈ 3 min if time permits, else reading

## Slide 24  —  Solved Example B — Cascade Impulse Response

- h⁽¹⁾ = (1,−1), h⁽²⁾ = (1,2,1): H = (1−z⁻¹)(1+2z⁻¹+z⁻²) = 1 + z⁻¹ − z⁻² − z⁻³, h = (1,1,−1,−1). Direct: h[0] = 1; h[1] = 2−1 = 1; h[2] = 1−2 = −1; h[3] = −1.

Timing: ≈ 3 min if time permits, else reading

## Slide 25  —  Common Mistakes

- 1 Not flipping (b₀ pairs with x[n], b₁ with x[n−1]). 2 Length is K+L−1, not K+L or max. 3 Terms exist only where 0 ≤ k ≤ K−1 and 0 ≤ n−k ≤ L−1. 4 DFT length N < K+L−1 wraps: (2,−4,7,−5) instead of the 7-sample result. 5 Overlap-add: add the K−1 shared samples. 6 Always check Σy = (Σh)(Σx) and the length.

Timing: ≈ 2 min

## Slide 26  —  Summary — Four Facts

- 01 impulse response defines an LTI system; 02 y[n] = Σh[k]x[n−k]; 03 length K+L−1; 04 overlap-add for long inputs.

- Bridge to L24: the product grid shows convolution is polynomial multiplication; next lecture names that polynomial the z-transform, H(z)X(z), and uses it to read filter specifications.

Timing: ≈ 1 min