ENES-222 · Signals & Systems

Lecture 4 Notes

Phasors & Discrete-Time Sinusoids

Thesis of the lecture: Two ideas, one splice point. First, the stationary phasor Ae^{jφ} compresses an entire same-frequency cosine into a single complex number, so adding sinusoids becomes adding phasors, with the time variable factored out. Then the lecture pivots from continuous time t to discrete sample index n: the frequency Ω is now radians per sample, and because n is an integer two extra equivalences appear (Ω ~ Ω+2π, and Ω ~ −Ω for real signals) that collapse the meaningful range to [0,π] and make periodicity a rational-number condition instead of automatic. Both halves matter for Lecture 5: sampling is exactly the machine that turns a CT ω into a DT Ω.

## Slide 1  —  Title — Phasors & Discrete-Time Sinusoids

- Convention for all three lectures: ω = continuous-time frequency (rad/s), Ω = discrete-time frequency (rad/sample). Say it aloud now.

- Timing: 80 min = 5 intro + 65 teaching + 5 break + 5 wrap. This deck is about 70 min if everything is used; slides 8 and 26 are optional (about 5 min) and can be skipped if you are behind.

## Slide 2  —  Objectives (1 min)

- Four objectives: phasor form of a sinusoid; add same-frequency sinusoids by adding phasors; Ω ≡ Ω+2π and Ω ≡ −Ω with effective range [0,π]; periodic iff Ω = (k/N)2π.

## Slide 3  —  Concept — Stationary Phasor (3 min)

- Goal: replace a cosine by ONE complex number that carries its amplitude and phase.

- Symbols (say them once): A = amplitude (real, ≥ 0); ω = continuous-time frequency in rad/s; t = time in s; φ = phase in rad; j = √−1 (so j² = −1); Re{z} = real part of z, e.g. Re{3 − 4j} = 3.

- Step 1 — Euler's formula: e^{jθ} = cos θ + j sin θ for any real angle θ (in radians). Its real part is cos θ, so cos θ = Re{e^{jθ}}.

- Step 2 — put θ = ωt + φ and multiply by the real number A (a real factor can go inside Re): A cos(ωt+φ) = Re{A e^{j(ωt+φ)}}.

- Step 3 — exponent rule e^{a+b} = e^a · e^b with a = jφ, b = jωt: A e^{j(ωt+φ)} = A e^{jφ} · e^{jωt}.

- Step 4 — so A cos(ωt+φ) = Re{A e^{jφ} e^{jωt}}. The factor A e^{jφ} does not contain t: it is the stationary phasor. e^{jωt} is the part that keeps turning.

- Meaning: z(t) = A e^{j(ωt+φ)} is an arrow of length A at angle ωt+φ; at t = 0 it is A e^{jφ}, the 'frozen' arrow.

- Tiny numeric example (the lecture numbers): A = 2.7, φ = 0.6, t = 0. Phasor = 2.7e^{j0.6} = 2.7(cos 0.6 + j sin 0.6) = 2.7(0.8253 + 0.5646j) = 2.2284 + 1.5245j. Its real part 2.2284 = 2.7 cos 0.6 = x(0) ✓.

## Slide 4  —  Visual — The Rotating Phasor (2 min)

- Data: A = 2.7, φ = 0.6 rad, ω = 15π rad/s = 47.124 rad/s. Plot x(t) = 2.7 cos(15πt + 0.6) = Re{z(t)}, z(t) = 2.7 e^{j(15πt+0.6)}.

- Step 1 — arrow angle at each time is θ = ωt + φ. t = 0: θ = 0.6 rad = 0.6(180/π) = 34.4°. t = 20 ms: ωt = 15π(0.02) = 0.3π = 0.9425 rad = 54°, so θ = 0.9425 + 0.6 = 1.5425 rad = 88.4°. t = 40 ms: ωt = 0.6π = 1.885 rad = 108°, so θ = 2.4850 rad = 142.4°.

- Step 2 — read the real axis (x = A cos θ). t = 0: 2.7 cos 0.6 = 2.7(0.8253) = 2.2284. t = 20 ms: 2.7 cos 1.5425 = 2.7(0.0283) = 0.0764 (arrow nearly vertical, so almost 0). t = 40 ms: 2.7 cos 2.4850 = 2.7(−0.7921) = −2.1385 (arrow in the second quadrant, real part negative).

- Step 3 — period: one lap = 2π rad, so T = 2π/ω = 2π/(15π) = 2/15 s = 133.3 ms. Frequency f = 1/T = 7.5 Hz.

- What to point at: the arrow's tip projected onto the real axis (left) is the dot on the curve (right) at the same height. Colours match: red t = 0, orange 20 ms, green 40 ms.

## Slide 5  —  Concept — Sum of Same-Frequency Sinusoids (3 min)

- Goal: add two cosines of the SAME ω and get one cosine. Notation: Re{·} = real part; P = a + jb has real part a and imaginary part b; |P| = √(a²+b²) is its length; ∠P is its angle.

- Line 1 — start: A₁cos(ωt+φ₁) + A₂cos(ωt+φ₂).

- Line 2 — write each cosine as a real part (slide 3): = Re{A₁e^{jφ₁}e^{jωt}} + Re{A₂e^{jφ₂}e^{jωt}}.

- Line 3 — Re{u} + Re{v} = Re{u+v} (real part of a sum is the sum of the real parts), then factor the common e^{jωt}: = Re{(A₁e^{jφ₁} + A₂e^{jφ₂}) e^{jωt}}.

- Line 4 — call the bracket A e^{jφ} and read it as a single cosine: = Re{A e^{jφ} e^{jωt}} = A cos(ωt+φ). So A e^{jφ} = A₁e^{jφ₁} + A₂e^{jφ₂}. The value of ω never entered the algebra.

- Back to polar, step 1 — rectangular form of each phasor: A_i e^{jφ_i} = A_i cos φ_i + j A_i sin φ_i. Add real parts together and imaginary parts together: P = a + jb.

- Back to polar, step 2 — A = |P| = √(a²+b²).

- Back to polar, step 3 — φ = ∠P = atan2(b, a). Definition: atan2(b,a) is the angle of the point (a,b) measured from the positive real axis, in (−π, π]; it picks the quadrant from the signs of a and b. Slide shows it with the numbers of the next slide: atan2(2.4561, −1.7644) = 2.1937, whereas a bare arctan(2.4561/(−1.7644)) = arctan(−1.392) = −0.9478, which is off by π.

- Quadrant rule if you use arctan: a > 0 → φ = arctan(b/a). a < 0, b ≥ 0 → φ = π − arctan(|b/a|). a < 0, b < 0 → φ = −π + arctan(|b/a|).

- Common error: Adding amplitudes: A ≠ A₁+A₂ unless the phases are equal. Check: A₁ = 2.7, A₂ = 4.1 give A = 3.02, not 6.8.

## Slide 6  —  Worked Example (1 of 2) (4 min)

- Given: x(t) = 2.7cos(15πt + 0.6) + 4.1sin(15πt − 1.8). Both terms have ω = 15π rad/s, so we may use phasors once the sine becomes a cosine.

- Step 1 — sine to cosine: sin θ = cos(θ − π/2) (a sine is a cosine delayed by a quarter turn). With θ = 15πt − 1.8: 4.1 sin(15πt−1.8) = 4.1 cos(15πt − 1.8 − π/2).

- Step 1 — the number: π/2 = 1.5708, so −1.8 − 1.5708 = −3.3708. Second term = 4.1cos(15πt − 3.3708).

- Step 2 — read off the phasors (A, φ): P₁ = 2.7e^{j0.6} and P₂ = 4.1e^{−j3.3708}.

- Step 2 — rectangular form a e^{jθ} = a cos θ + j a sin θ. Needed values: cos 0.6 = 0.8253, sin 0.6 = 0.5646; cos(−3.3708) = −0.9738, sin(−3.3708) = +0.2272 (the angle −3.3708 rad = −193.1° is in the second quadrant: cos negative, sin positive).

- P₁: real part 2.7(0.8253) = 2.2284; imaginary part 2.7(0.5646) = 1.5245. So P₁ = 2.2284 + 1.5245j.

- P₂: real part 4.1(−0.9738) = −3.9928; imaginary part 4.1(0.2272) = 0.9315. So P₂ = −3.9928 + 0.9315j.

- Check P₂ another way: 4.1 sin(−1.8) at t = 0 should equal Re P₂. sin(−1.8) = −0.9738, 4.1(−0.9738) = −3.9928 ✓ (the same number, since cos(−3.3708) = sin(−1.8)).

- Common error: Forgetting to convert sin to cos before forming the phasor, or adding π/2 instead of subtracting.

## Slide 7  —  Worked Example (2 of 2) (4 min)

- Continue with P₁ = 2.2284 + 1.5245j and P₂ = −3.9928 + 0.9315j.

- Step 3 — add: real part 2.2284 + (−3.9928) = −1.7644. Imaginary part 1.5245 + 0.9315 = 2.4560 (2.4561 with unrounded numbers). P = −1.7644 + 2.4561j.

- Step 4 — amplitude: a² = (−1.7644)² = 3.1131; b² = (2.4561)² = 6.0324; sum = 9.1455; A = √9.1455 = 3.0241 (carry the unrounded numbers; the exact value is 3.02411).

- Step 4 — angle: signs a < 0, b > 0, so P is in the second quadrant. b/|a| = 2.4561/1.7644 = 1.3920; arctan(1.3920) = 0.9478 rad (54.3°). φ = π − 0.9478 = 3.1416 − 0.9478 = 2.1937 rad (125.7°). Same as atan2(2.4561, −1.7644).

- Answer: x(t) = 3.0241 cos(15πt + 2.1937).

- Check 1 (t = 0): original 2.7cos 0.6 + 4.1 sin(−1.8) = 2.2284 + 4.1(−0.9738) = 2.2284 − 3.9928 = −1.7644. Answer: 3.0241 cos 2.1937 = 3.0241(−0.5834) = −1.7643 ✓ (4-digit rounding).

- Check 2 (t = 0.1 s): original = 2.4561; answer = 3.0241 cos(1.5π + 2.1937) = 2.4561 ✓ (notebook difference ≈ 1e−14).

- Check 3 (size): A = 3.0241 < 2.7 + 4.1 = 6.8, so the arrows partly cancel. FIGURE: the dashed red curve lies on the blue sum; its peak is about 3.02, not 6.8.

## Slide 8  —  Visual — Phasor Sum as a Vector Sum (2 min, optional)

- Goal: the same example as an arrow picture. Arrow = complex number; lengths |P₁| = 2.7, |P₂| = 4.1.

- Step 1 — angle of P₁ = 0.6 rad = 34.4°.

- Step 2 — angle of P₂ = −3.3708 rad = −193.1°. Add a full turn (360°): 166.9° (same arrow, because e^{j2π} = 1). Its coordinates: 4.1(cos 166.9°, sin 166.9°) = (−3.9928, 0.9315) ✓.

- Step 3 — angle between the arrows: 166.9° − 34.4° = 132.5° (more than 90°, so they pull against each other).

- Step 4 — head-to-tail: slide P₂ (dashed green) so its tail sits on the tip of P₁; the arrow from the origin to the new tip (red) is P = P₁ + P₂.

- Step 5 — numbers on the red arrow: P = −1.7644 + 2.4561j, |P| = 3.0241, angle 125.7° = 2.1937 rad (second quadrant). |P₁| + |P₂| = 6.8 would only happen if both pointed the same way.

## Slide 9  —  Practice — Add Two Sinusoids (3 min)

- Task for the students (3 min): (a) 4cos(10πt) + 3sin(10πt); (b) 2cos(7t+2) + 2cos(7t−1).

- Recipe to write on the board: 1) sine → cosine (sin θ = cos(θ − π/2)); 2) each phasor A e^{jφ} = A cos φ + jA sin φ, then add; 3) A = |P|, φ = atan2(b,a) with the quadrant from the signs.

- Hints if they stall: (a) the frequency is 10π for both; the phasors are 4e^{j0} and 3e^{−jπ/2}. (b) both are already cosines; angles are in radians, so use a calculator in radian mode.

## Slide 10  —  Solution (a) (2 min)

- Given: 4cos(10πt) + 3sin(10πt); both have ω = 10π rad/s.

- Step 1 — sin(10πt) = cos(10πt − π/2), so the second term is 3cos(10πt − π/2).

- Step 2 — phasors: P₁ = 4e^{j0} = 4. P₂ = 3e^{−jπ/2} = 3(cos(π/2) − j sin(π/2)) = 3(0 − j·1) = −3j.

- Step 3 — add: P = 4 + (−3j) = 4 − 3j (real part 4, imaginary part −3).

- Step 4 — A = √(4² + (−3)²) = √(16 + 9) = √25 = 5.

- Step 4 — φ: real part positive, so P is in the fourth quadrant and the principal value is correct: φ = arctan(−3/4) = arctan(−0.75) = −0.6435 rad = −36.87°.

- Answer: 5cos(10πt − 0.6435).

- Check t = 0: original 4cos 0 + 3 sin 0 = 4 + 0 = 4; answer 5cos(−0.6435) = 5(0.8) = 4 ✓. Check t = 0.05 s: original 4cos(π/2) + 3sin(π/2) = 0 + 3 = 3; answer 5cos(π/2 − 0.6435) = 5 sin(0.6435) = 5(0.6) = 3 ✓.

- Familiar 3-4-5 triangle: cos φ = 4/5 = 0.8, sin φ = −3/5 = −0.6.

## Slide 11  —  Solution (b) (3 min)

- Given: 2cos(7t+2) + 2cos(7t−1); ω = 7 rad/s for both, both already cosines. All angles in radians.

- Step 1 — P₁ = 2e^{j2} = 2cos 2 + j 2sin 2. cos 2 = −0.4161, sin 2 = 0.9093. So P₁ = 2(−0.4161) + j2(0.9093) = −0.8323 + 1.8186j.

- Step 2 — P₂ = 2e^{−j1} = 2cos 1 − j 2sin 1. cos 1 = 0.5403, sin 1 = 0.8415. So P₂ = 2(0.5403) − j2(0.8415) = 1.0806 − 1.6829j.

- Step 3 — add: real −0.8323 + 1.0806 = 0.2483; imaginary 1.8186 − 1.6829 = 0.1357. P = 0.2483 + 0.1357j.

- Step 4 — A = √(0.2483² + 0.1357²) = √(0.06165 + 0.01841) = √0.08006 = 0.2829.

- Step 4 — φ: both parts positive, first quadrant: φ = arctan(0.1357/0.2483) = arctan(0.5465) = 0.5000 rad.

- Answer: 0.2829cos(7t + 0.5).

- Check t = 0: 2cos 2 + 2cos(−1) = −0.8323 + 1.0806 = 0.2483; answer 0.2829cos 0.5 = 0.2829(0.8776) = 0.2483 ✓.

- Second method (equal amplitudes): 2e^{j2} + 2e^{−j1} = 2e^{j0.5}(e^{j1.5} + e^{−j1.5}) = 2e^{j0.5}·2cos 1.5 = 4cos(1.5) e^{j0.5}. cos 1.5 = 0.0707, so A = 4(0.0707) = 0.2829 ✓ and φ = 0.5 = (2 + (−1))/2 ✓. Here e^{jθ} + e^{−jθ} = 2cos θ.

- Insight: the phases 2 and −1 are 3 rad (172°) apart, nearly opposite, so the sum is tiny.

## Slide 12  —  Concept — Discrete-Time Sinusoid (2 min)

- Definition: x[n] = A cos(Ωn + φ). n = 0, 1, 2, … is an integer sample number (no unit); Ω = DT frequency in rad per sample (how much the angle grows from one sample to the next); A, φ as before. Capital Ω = discrete time, lowercase ω = continuous time.

- Tiny numeric example: A = 1, Ω = π/2, φ = 0. Angles Ωn = 0, π/2, π, 3π/2, 2π, … so x[n] = cos(Ωn) = 1, 0, −1, 0, 1, … (a pattern of 4 samples).

- There is no t and no seconds in the formula: only counting samples.

## Slide 13  —  Visual — What DT Sinusoids Look Like (2 min)

- FIGURE: four panels, all x[n] = A cos(Ωn + φ) with different Ω. Numbers behind each panel:

- Top left Ω = π/25 = 0.1257 rad/sample, φ = 2π/5: angle grows by 0.1257 each sample. x[n] = cos(0.1257n + 1.2566): x[0] = 0.3090, x[1] = 0.1874, x[2] = 0.0628, x[3] = −0.0628, x[4] = −0.1874: a slow, smooth wave. Samples per cycle = 2π/Ω = 2π/(π/25) = 50.

- Top right Ω = 2.4 rad/sample, φ = −1.3: x[0] = cos(−1.3) = 0.2675, x[1] = cos(1.1) = 0.4536, x[2] = cos(3.5) = −0.9365, x[3] = cos(5.9) = 0.9275: jumps around. 2.4 is close to π (π − 2.4 = 0.742), so the angle advances almost half a turn each sample and the eye cannot follow.

- Bottom left Ω = 24π/25 = 3.0159 = π − π/25: x[0] = 0.3090, x[1] = −0.4258, x[2] = 0.5358, x[3] = −0.6374: the sign flips every sample while the size grows slowly (a slow envelope). Reason: cos((π − π/25)n + φ) = (−1)ⁿ cos((π/25)n − φ).

- Bottom right: Ω = 0 gives cos(φ) every sample (constant); Ω = π gives cos(πn) = +1, −1, +1, −1, … (alternating).

- Point at: low Ω looks smooth; Ω near π looks jagged; the panel titles use the capital letter Ω.

## Slide 14  —  Convention — Frequency Equivalence (3 min)

- Goal: show which DT frequencies describe the same samples. Notation: a ≡ b (mod 2π) means a − b is a whole number of full turns (an integer multiple of 2π); e.g. 7π/4 ≡ −π/4 because 7π/4 − (−π/4) = 2π.

- Rule 1 (complex or real) — step 1: e^{j(Ω+2π)n} = e^{jΩn} · e^{j2πn} (exponent rule e^{a+b} = e^a e^b).

- Rule 1 — step 2: e^{j2πn} = cos 2πn + j sin 2πn = 1 + 0 = 1 for every integer n (n whole turns). So e^{j(Ω+2π)n} = e^{jΩn}: Ω and Ω + 2π give identical samples. Example n = 3: e^{j2π·3} = 1.

- Rule 2 (real signals only) — cos is even: cos(θ) = cos(−θ). With θ = Ωn + φ: cos(Ωn+φ) = cos(−Ωn−φ). So (Ω, φ) and (−Ω, −φ) give identical samples: BOTH signs change.

- Effective range: any Ω can be moved into [0, π]. Ω = 0 is the lowest (DC), Ω = π is the highest (alternating).

- Recipe step 1 — subtract multiples of 2π until 0 ≤ Ω < 2π.

- Recipe step 2 — if Ω > π, replace Ω by 2π − Ω and negate φ. Why: Ω → −Ω, φ → −φ (rule 2), then add 2π to the frequency (rule 1): cos(Ωn+φ) = cos(−Ωn−φ) = cos((2π−Ω)n − φ).

- Complex exponentials: only rule 1 applies (there is no sign flip), so they are reduced to [0, 2π) only.

## Slide 15  —  Worked Example — Reduce to the Standard Range (4 min)

- Complex case: z[n] = e^{j((37π/13)n + π/8)}.

- Step 1 — size: 37π/13 = 2.846π ≈ 8.94 rad, larger than 2π.

- Step 2 — subtract a full turn, 2π = 26π/13: 37π/13 − 26π/13 = 11π/13 (≈ 0.846π, inside [0, 2π)). Allowed because e^{−j2πn} = 1.

- Answer: z[n] = e^{j((11π/13)n + π/8)}. The phase π/8 is untouched; no sign flip for complex exponentials.

- Real case: x[n] = cos((42π/13)n − 5π/6).

- (a) Subtract 2π = 26π/13: 42π/13 − 26π/13 = 16π/13 = 1.231π. Still larger than π.

- (b) Evenness of cos: x[n] = cos(−(16π/13)n + 5π/6) (frequency and phase both change sign).

- (c) Add 2π to the frequency: −16π/13 + 26π/13 = 10π/13 = 0.769π, inside [0, π]. Phase stays 5π/6.

- Answer: x[n] = cos((10π/13)n + 5π/6). Same as the slide-14 recipe: 2π − 16π/13 = 10π/13, phase −(−5π/6) = +5π/6.

- Check n = 1: original cos(42π/13 − 5π/6) = cos(10.1498 − 2.6180) = cos(7.5318) ≈ 0.3167 (use the unrounded angle on the calculator). New: cos(10π/13 + 5π/6) = cos(2.4166 + 2.6180) = cos(5.0346) = 0.3167 ✓. Check n = 2: both 0.3920 ✓.

## Slide 16  —  Practice — Endpoints (2 min)

- Task (1 min): what does x[n] = A cos(Ωn + φ) look like at the two ends of the range, Ω = 0 and Ω = π?

- Hint: put the value in, and for π use cos(a+b) = cos a cos b − sin a sin b.

## Slide 17  —  Solution — Endpoints (2 min)

- Ω = 0 (lowest): x[n] = A cos(0·n + φ) = A cos φ. The argument does not change with n, so every sample equals A cos φ: a constant. Example A = 2, φ = π/3: x[n] = 2(0.5) = 1 for all n.

- Ω = π (highest), step 1 — angle addition with a = πn, b = φ: cos(πn + φ) = cos(πn) cos φ − sin(πn) sin φ.

- Step 2 — for integer n: cos(πn) = (−1)ⁿ (n = 0, 1, 2, 3 → 1, −1, 1, −1) and sin(πn) = 0.

- Step 3 — so x[n] = A (−1)ⁿ cos φ − A·0·sin φ = A(−1)ⁿ cos φ: the samples go +, −, +, −, … times A cos φ. Example A = 2, φ = π/3: x[n] = 1, −1, 1, −1, …

- Meaning: the angle jumps by π (half a turn) every sample; that is the fastest oscillation a DT sinusoid can have.

## Slide 18  —  Practice — Reduce the Frequency (3 min)

- Task (3 min): rewrite each with Ω ∈ [0, π] and the same samples: (a) cos(11π/6 n + π/4); (b) 3cos(9.5n + 0.2); (c) 2cos(5n − 1).

- Recipe on the board: (1) subtract 2π's until Ω ∈ [0, 2π); (2) if Ω > π, Ω → 2π − Ω and φ → −φ; (3) check with n = 3.

- Things students trip on: 9.5 is in radians (not degrees), and 9.5 > 2π = 6.2832, so a subtraction is needed in (b) but not in (a) or (c).

## Slide 19  —  Solution — Reduce the Frequency (3 min)

- (a) Ω = 11π/6 = 5.760, already in [0, 2π) but larger than π (= 3.1416). Step 1: 2π − 11π/6 = 12π/6 − 11π/6 = π/6. Step 2: negate the phase: π/4 → −π/4. Answer cos((π/6)n − π/4).

- Check (a), n = 3: original cos(11π/6·3 + π/4) = cos(5.5π + 0.25π) = cos(5.75π); subtract two full turns (4π): cos(1.75π) = cos(−0.25π) = 0.7071. New: cos(π/6·3 − π/4) = cos(π/2 − π/4) = cos(π/4) = 0.7071 ✓.

- (b) Ω = 9.5. Step 1: 9.5 − 2π = 9.5 − 6.2832 = 3.2168, which is larger than π = 3.1416. Step 2: 2π − 3.2168 = 6.2832 − 3.2168 = 3.0664. Step 3: negate the phase 0.2 → −0.2. Answer 3cos(3.0664n − 0.2).

- Check (b), n = 3: original 3cos(28.5 + 0.2) = 3cos(28.7) = 3(−0.9108) = −2.7323. New: 3cos(9.1992 − 0.2) = 3cos(8.9992) = 3(−0.9108) = −2.7323 ✓. Remark: 3.0664 is just below π, so the signal is nearly alternating.

- (c) Ω = 5 is in [0, 2π) but larger than π. Step 1: 2π − 5 = 6.2832 − 5 = 1.2832. Step 2: negate the phase −1 → +1. Answer 2cos(1.2832n + 1).

- Check (c), n = 3: original 2cos(15 − 1) = 2cos 14 = 2(0.1367) = 0.2735. New: 2cos(3.8496 + 1) = 2cos(4.8496) = 0.2735 ✓.

- Common error: Reducing Ω but forgetting to negate φ. Test: in (c) without the sign change you would get 2cos(1.2832·3 − 1) = 2cos(2.8496) = −1.92 ≠ 0.2735.

## Slide 20  —  Concept — Periodicity of x[n] (3 min)

- Definition: x[n] is periodic with period N (a positive integer) if x[n+N] = x[n] for every n. The fundamental period is the smallest such N.

- Step 1 — x[n+N] = cos(Ω(n+N) + φ) = cos(Ωn + φ + ΩN).

- Step 2 — this equals cos(Ωn + φ) for every n exactly when ΩN is a whole number of turns: ΩN = 2πk with k an integer.

- Step 3 — rewrite: Ω = (k/N)·2π, i.e. Ω/2π must be a ratio of two integers (rational).

- Warning: the fundamental period is NOT simply 2π/Ω, which is usually not an integer. Example Ω = 3π/7: 2π/Ω = 14/3 = 4.67, but N = 14.

- If Ω/2π is irrational (for instance Ω = 1, since 1/(2π) = 0.159 involves π): no integer N works, x[n] is aperiodic although it still looks wavy.

## Slide 21  —  Derivation — N Is the Denominator (3 min)

- Goal: prove N = denominator of Ω/2π in lowest terms. Write Ω/2π = p/q with integers p, q having no common factor (lowest terms).

- Step 1 — periodic means ΩN = 2πm for an integer m (slide 20).

- Step 2 — substitute Ω = 2πp/q: (2πp/q)N = 2πm. Divide by 2π: pN/q = m. So pN/q must be an integer, i.e. q divides pN.

- Step 3 — p and q share no factor, so q cannot get its factors from p. Hence q divides N itself: N = q, 2q, 3q, …

- Step 4 — smallest choice N = q; then m = pN/q = p works.

- Example Ω = 0.3π: Ω/2π = 0.15 = 15/100 = 3/20 (divide top and bottom by 5). So N = 20, k = 3. Check: ΩN = 0.3π(20) = 6π = 2π·3 ✓.

- Why not N = 10? pN/q = 3(10)/20 = 1.5, not an integer: 0.3π(10) = 3π is only 1.5 turns, so the pattern has not come back to the same phase.

- If Ω/2π is irrational there is no fraction p/q at all, so no N exists.

## Slide 22  —  Examples — Fundamental Periods (2 min)

- Method for each row: form Ω/2π, reduce to lowest terms k/N, read N.

- Ω = 0: Ω/2π = 0/1, N = 1 (constant sequence).

- Ω = π: Ω/2π = 1/2, N = 2 (alternates). Check: πN = 2π = 2π·1 ✓.

- Ω = 1.0: Ω/2π = 1/(2π) = 0.159…, which is irrational (π is irrational): aperiodic.

- Ω = 10π/13: Ω/2π = 10π/(13·2π) = 10/26 = 5/13 (divide by 2), lowest terms, so N = 13, k = 5. Check: ΩN = 10π = 2π·5 ✓.

- Ω = 11π/13: Ω/2π = 11/26, already in lowest terms (11 is prime and does not divide 26), so N = 26, k = 11, not 13. Check: ΩN = 11π·26/13 = 22π = 2π·11 ✓. Test N = 13: 11π = 5.5 turns ✗.

## Slide 23  —  Visual — Periodic vs Aperiodic (2 min)

- FIGURE: stem plots of cos(Ωn), n = 0…49, four values of Ω. Numbers behind them:

- Blue Ω = 0: cos 0 = 1 for every n (N = 1). Red Ω = π: 1, −1, 1, −1, … (N = 2).

- Green Ω = 10π/13: one lap of 13 samples. x[0] = 1, x[1] = −0.7485, x[2] = 0.1205, x[3] = 0.5681, x[4] = −0.9709, x[5] = 0.8855, x[6] = −0.3546, x[7] = −0.3546, … and x[13] = 1. Because ΩN = 10π = 2π·5, the angle at n = 13 is five full turns, so x[13] = x[0] = 1; same at n = 26, 39 (all equal 1).

- Orange Ω = 1.0: 1, 0.5403, −0.4161, −0.9900, −0.6536, 0.2837, 0.9602, 0.7539, … It never returns exactly to 1 (that would need 1·n = 2πk, impossible since π is irrational).

- What to point at: in the green panel compare samples 0, 13, 26, 39 (all at height 1); the orange pattern never repeats.

## Slide 24  —  Practice — Fundamental Periods (3 min)

- Task (3 min): find N or say aperiodic: (1) cos(0.3πn); (2) cos(7π/4 n + 0.5); (3) cos(3π/7 n); (4) cos(3n).

- Method on the board: write Ω/2π, simplify the fraction, read the denominator, then verify ΩN = 2πk.

- Hint for (2): the phase 0.5 plays no role in periodicity. Hint for (4): Ω = 3 rad/sample has no π in it.

## Slide 25  —  Solution — Fundamental Periods (2 min)

- (1) Ω = 0.3π: Ω/2π = 0.3/2 = 0.15 = 3/20, so N = 20. Check ΩN = 0.3π(20) = 6π = 2π·3 ✓.

- (2) Ω = 7π/4 (the phase 0.5 does not matter): Ω/2π = 7/8, lowest terms, so N = 8. Check: (7π/4)(8) = 14π = 2π·7 ✓.

- (3) Ω = 3π/7: Ω/2π = 3/14, lowest terms, so N = 14. Check: (3π/7)(14) = 6π = 2π·3 ✓. Not 2π/Ω = 14/3 = 4.67 (not an integer).

- (4) Ω = 3: Ω/2π = 3/(2π) = 0.4775…, irrational (contains π), so aperiodic. Check some N: 3N = 2πk would need π = 3N/(2k), a ratio of integers, impossible. Numerically 3N/(2π) for N = 1, 2, 3, … = 0.477, 0.955, 1.432, 1.910, 2.387, … never an integer.

## Slide 26  —  Application — Balanced Three-Phase (3 min, optional)

- Data: three voltages with the same ω, shifted by a third of a turn: v_k(t) = cos(ωt − 2πk/3), k = 0, 1, 2. Claim: v₀ + v₁ + v₂ = 0 for every t.

- Step 1 — phasors (A = 1, φ = −2πk/3): P₀ = 1; P₁ = e^{−j2π/3}; P₂ = e^{−j4π/3}.

- Step 2 — rectangular form: 2π/3 = 120°, cos 120° = −1/2, sin 120° = √3/2 = 0.866. So P₁ = cos(2π/3) − j sin(2π/3) = −1/2 − j√3/2. 4π/3 = 240°, cos 240° = −1/2, sin 240° = −√3/2, so P₂ = cos(4π/3) − j sin(4π/3) = −1/2 + j√3/2.

- Step 3 — add real parts: 1 − 1/2 − 1/2 = 0. Add imaginary parts: 0 − √3/2 + √3/2 = 0. So P = P₀ + P₁ + P₂ = 0.

- Step 4 — v₀ + v₁ + v₂ = Re{P e^{jωt}} = Re{0} = 0 for every t (notebook: max |sum| ≈ 1e−15).

- Picture: three unit arrows 120° apart, placed head to tail, close into an equilateral triangle (sum = 0). Equal loads: the three currents cancel, so the neutral wire carries nothing.

## Slide 27  —  Common Mistakes (2 min)

- Six mistakes, one numeric example each:

- 01 Amplitudes: 2.7 and 4.1 give A = 3.02, not 6.8 (add phasors, then take |P|).

- 02 Sine: sin θ = cos(θ − π/2); e.g. sin 0.3 = 0.2955 = cos(0.3 − 1.5708) = cos(−1.2708) = 0.2955. Adding π/2 would give cos(1.8708) = −0.2955, wrong.

- 03 Quadrant: P = −1.7644 + 2.4561j: arctan(−1.392) = −0.948 (fourth quadrant) but the true angle is 2.1937 (second quadrant).

- 04 Phase flip: when Ω → 2π − Ω the phase must change sign: e.g. 2cos(5n − 1) → 2cos(1.2832n + 1), not 2cos(1.2832n − 1).

- 05 Period: N is the denominator of Ω/2π in lowest terms, not 2π/Ω: for 3π/7, 2π/Ω = 4.67 but N = 14.

- 06 Symbols: ω is rad/s (continuous time), Ω is rad/sample (discrete time); there are no seconds in x[n].

## Slide 28  —  Summary and Bridge (2 min)

- Four facts: phasor; DT sinusoid; equivalence/range; periodicity.

- Bridge: next lecture samples a CT sinusoid every T_s seconds, with Ω = ωT_s.